# iiot technologies

# Integration of OPTIFY™ platform with FactoryTalk Optix Studio

Step-by-step guide for customers

Prepared On:

July 21, 2025

Contents

Executive summary	4

Prerequisites	5

Step-by-step guide	6

Step 1: Set up the environment	6

Step 2: Run the FactoryTalk scripts	7

Notes	10

Troubleshooting	11

Revision history	12

# Executive summary

This document provides a step-by-step guide to the integration of the OPTIFY platform by Dodge® Industrial for customers.

Dodge OPTIFY integration with SCADA - Setup and Usage Guide

This guide provides step-by-step instructions to set up the environment and run the Ignition scripts in FactoryTalk Optix Studio.

# Prerequisites

- FactoryTalk Optix Studio:

  - Install FactoryTalk Optix Studio from Rockwell Automation

- API Key and Plant ID:

  - Obtain your API Key by following this guide:
Add New API Key

  - Find your Plant ID by following this guide:
Find the Plant ID

# Step-by-step guide

## Step 1: Set up the environment

- Prepare your API Key and Plant ID

- Open Windows System Environment Variables Panel

- Create a new system variable with the name OPTIFY_API_KEY and your API Key as the variable value

## Step 2: Run the FactoryTalk scripts

- Open FactoryTalk Optix Studio and open your project

- In the NetLogic folder, create two Design-time NetLogic scripts, with the corresponding names

- Open GenerateTagTree.cs and change PLANT_ID to the desired Plant ID. Copy the whole script.

- Copy the code into FactoryTalk Optix Studio

- Wait for it to build, right-click the script, and click Execute BuildTagTree

- In the security warning window, click Proceed

- Wait for tags to generate

- Open the UpdateKPIValues.cs script

- Copy the Root tag name and paste it into the script

- Mark all content of the file and copy the script, paste it into FactoryTalk Optix Studio

- Right-click the script and click Execute PopulateValues

- All variables should update shortly after

# Notes

- Ensure that the API Key in the Environment Variable and Plant ID are valid and that the Optix and NetLogic have network access to the Dodge OPTIFY API

# Troubleshooting

- If the script fails to authenticate, verify the API Key in the Environment Variable and the Plant ID in the script

- Check the Studio Output logs for any errors related to the script execution

By following these steps, you should be able to set up the environment and run the NetLogic scripts successfully.

# Revision history

| Document name: Integration of Dodge OPTIFY Platform with FactoryTalk Optix Studio | Status: Released | Security level: External |
| --- | --- | --- |
| Date of revision: 07/21/2025 | Revision number: 1 | Page(s): 12 |
| Author(s): Maciej Grochowski | Revised by: Przemyslaw Piatek | Approved by: Artur Poniedzialek |

| Revision | Date | Author | Description |
| --- | --- | --- | --- |
| 1 | 21st July 2025 | Maciej Grochowski | Initial release |