# iiot technologies

# Integration of the OPTIFY™ platform with Ignition Designer

Step-by-step guide for customers

Prepared On:

July 21, 2025

Contents

Executive summary	4

Prerequisites	5

Step-by-step guide	6

Step 1: Set up the environment	6

Step 2: Run the Ignition scripts	8

Notes	15

Troubleshooting	16

Revision history	17

# Executive summary

This document provides a step-by-step guide to the integration of the OPTIFY platform by Dodge® Industrial for customers.

Dodge OPTIFY integration with SCADA - Setup and Usage Guide

This guide provides step-by-step instructions to set up the environment and run the Ignition scripts in Ignition Designer.

# Prerequisites

- Ignition Designer:

  - Install Ignition Designer from Inductive Automation

- API Key and Plant ID:

  - Obtain your API Key by following this guide:
Add New API Key

  - Find your Plant ID by following this guide:
Find the Plant ID

# Step-by-step guide

## Step 1: Set up the environment

- Prepare your API Key and Plant ID

- Open Windows Environment Variables panel

- Create a new system variable with the name OPTIFY_API_KEY and your API Key as the variable value

- Now we need to restart the Ignition service, restart the computer, or open a Command Prompt as Administrator and run these commands

## Step 2: Run the Ignition scripts

- Open Ignition Designer and connect to your Ignition Gateway

- Sign in by providing your username and password

- Click the ‘Open’ button

- Open the Scripting section in Ignition Designer

- Open ignition_startup_create_tags.py and change PLANT_ID to the desired Plant ID. Copy the whole script.

- Paste the code into Startup

- Click OK and save the project

- Wait for tags to generate

- Open the ignition_timer_get_measurements.py script located in the Ignition scripts folder

- Copy the Root tag name and paste it into the script

- Select all the content in the file and copy the script

- Open the Scripting section in Ignition Designer and switch to the Timer window

- Create a Gateway Timer by setting its name and delay milliseconds value

- Apply changes to the timer and save the project

- Start the Ignition Gateway and monitor the logs to ensure the script runs successfully

- After the Gateway timeout, you should be able to see proper values assigned

# Notes

- The ignition_timer_get_measurements.py script fetches the latest measurement values for sensors and writes them to the appropriate tags in Ignition.

- Ensure that the API Key in the Environment Variable and Plant ID are valid and that the Ignition Gateway has network access to the Dodge OPTIFY API.

# Troubleshooting

- If the script fails to authenticate, verify the API Key in the Environment Variable and the Plant ID in the script.

- Check the Ignition Gateway logs for any errors related to the script execution.

By following these steps, you should be able to set up the environment and run the Ignition scripts successfully.

# Revision history

| Document name: Integration of Dodge OPTIFY Platform with SCADA | Status: Released | Security level: External |
| --- | --- | --- |
| Date of revision: 07/21/2025 | Revision number: 3 | Page(s): 17 |
| Author(s): Przemyslaw Piatek, Maciej Grochowski | Revised by: Przemyslaw Piatek | Approved by: Artur Poniedzialek |

| Revision | Date | Author | Description |
| --- | --- | --- | --- |
| 1 | 8th May 2025 | Przemyslaw Piatek, Maciej Grochowski | Initial release |
| 2 | 29th May 2025 | Przemyslaw Piatek, Maciej Grochowski | Display improvement |
| 3 | 21st July 2025 | Maciej Grochowski | Environment variables usage without python installation process |